Assignment 4

Ameer Ashraf Louly – G3

Q1: ALSU

RTL Code:

module alsu_seq(A, B, opcode, cin, serial_in, direction, red_op_A, red_op_B, bypass_A, bypass_B, clk, rst,  out, leds);

output reg [5:0] out;

output reg [15:0] leds;

parameter INPUT_PRIORITY = "A";

parameter FULL_ADDER = "ON";

input [2:0] A;

input [2:0] B;

input [2:0] opcode;

input   cin,

serial_in,

direction,

red_op_A,

red_op_B,

bypass_A,

bypass_B,

clk,

rst;

reg Invalid_Case;

reg cin_reg;

reg serial_in_reg;

reg direction_reg;

reg red_op_A_reg;

reg red_op_B_reg;

reg bypass_A_reg;

reg bypass_B_reg;

reg [2:0] A_reg;

reg [2:0] B_reg;

reg [2:0] opcode_reg;

// Inputs Always Block

always @(posedge clk or posedge rst) begin

if(rst) begin

cin_reg <= 0;

serial_in_reg <= 0;

direction_reg <= 0;

red_op_A_reg <= 0;

red_op_B_reg <= 0;

bypass_A_reg <= 0;

bypass_B_reg <= 0;

A_reg <= 0;

B_reg <= 0;

opcode_reg <= 0;

end

else begin

cin_reg <= cin;

serial_in_reg <= serial_in;

direction_reg <= direction;

red_op_A_reg <= red_op_A;

red_op_B_reg <= red_op_B;

bypass_A_reg <= bypass_A;

bypass_B_reg <= bypass_B;

A_reg <= A;

B_reg <= B;

opcode_reg <= opcode;

end

end

// LEDs Always Block

always @(posedge clk or posedge rst) begin

if(rst)

leds <= 0;

else if(Invalid_Case == 1)

leds <= ~leds;

else

leds <= 0;

end

always @(posedge clk or posedge rst) begin

if(rst) begin

out <= 0;

end

else if(bypass_A_reg || bypass_B_reg) begin

if(bypass_A_reg && bypass_B_reg) begin

if(INPUT_PRIORITY == "A")

out <= A_reg;

if(INPUT_PRIORITY == "B")

out <= B_reg;

end

else if(bypass_A_reg)

out <= A_reg;

else if(bypass_B_reg)

out <= B_reg;

end

else begin

case(opcode_reg)

3'h0: begin

if(red_op_A_reg == 1 && red_op_B_reg == 1) begin

if(INPUT_PRIORITY == "A")

out <= &A_reg;

else if(INPUT_PRIORITY == "B")

out <= &B_reg;

end else if (red_op_A_reg == 1)

out <= &A_reg;

else if(red_op_B_reg == 1)

out <= &B_reg;

else begin

out <= A_reg & B_reg;

end

Invalid_Case <= 0;

end

3'h1: begin

if(red_op_A_reg && red_op_B_reg) begin

if(INPUT_PRIORITY == "A")

out <= ^A_reg;

else if(INPUT_PRIORITY == "B")

out <= ^B_reg;

end

else if (red_op_A_reg)

out <= ^A_reg;

else if(red_op_B_reg)

out <= ^B_reg;

else begin

out <= A_reg ^ B_reg;

end

Invalid_Case <= 0;

end

3'h2: begin

if (red_op_A_reg || red_op_B_reg)

Invalid_Case <= 1;

else if(FULL_ADDER == "ON") begin

out <= A_reg + B_reg + cin_reg;

Invalid_Case <= 0;

end

else if (FULL_ADDER == "OFF") begin

out <= A_reg + B_reg;

Invalid_Case <= 0;

end

end

3'h3: begin

if (red_op_A_reg || red_op_B_reg)

Invalid_Case <= 1;

else begin

out <= A_reg * B_reg;

Invalid_Case <= 0;

end

end

3'h4: begin

if (red_op_A_reg || red_op_B_reg)

Invalid_Case <= 1;

else if(direction_reg) begin

out <= {out[4:0], serial_in_reg};

Invalid_Case <= 0;

end

else begin

out <= {serial_in_reg, out[5:1]};

Invalid_Case <= 0;

end

end

3'h5: begin

if (red_op_A_reg || red_op_B_reg)

Invalid_Case <= 1;

else if(direction_reg) begin

out <= {out[4:0], out[5]};

Invalid_Case <= 0;

end

else begin

out <= {out[0], out[5:1]};

Invalid_Case <= 0;

end

end

3'h6: Invalid_Case <= 1;

3'h7: Invalid_Case <= 1;

endcase

end

end

endmodule

Testbench Code:

module alsu_tb();

reg [2:0] A, B, opcode;

reg cin,

serial_in,

direction,

red_op_A,

red_op_B,

bypass_A,

bypass_B,

clk,

rst;

reg [5:0] out_expected;

wire [5:0] out;

wire [15:0] leds;

alsu_seq DUT(A,

B,

opcode,

cin,

serial_in,

direction,

red_op_A,

red_op_B,

bypass_A,

bypass_B,

clk,

rst,

out,

leds);

initial begin

clk = 0;

forever begin

#1 clk = ~clk;

end

end

initial begin

// Reset Functionality

A = 0;

B = 0;

opcode = 0;

cin = 0;

serial_in = 0;

direction = 0;

red_op_A = 0;

red_op_B = 0;

bypass_A = 0;

bypass_B = 0;

rst = 1;

repeat(2) @(negedge clk)

if(out != 0 && leds != 0) begin

$display("Error - Reset");

$exit;

end

// bypass functionality

rst = 0;

bypass_A = 1;

bypass_B = 1;

repeat(50) begin

A = $random;

B = $random;

opcode =  $urandom_range(0, 5);

out_expected = A;

repeat(2) @(negedge clk);

if(out != out_expected) begin

$display("Error - Bypass");

$exit;

end

end

rst = 1;

repeat(2) @(negedge clk)

// Opcode = 0 functionality

rst = 0;

A = 0;

B = 0;

bypass_A = 0;

bypass_B = 0;

opcode = 0;

repeat(2) @(negedge clk)

repeat(50) begin

A = $random;

B = $random;

red_op_A = $random;

red_op_B = $random;

if(red_op_A && red_op_B)

out_expected = &A;

else if(red_op_A)

out_expected = &A;

else if(red_op_B)

out_expected = &B;

else

out_expected = A & B;

repeat(2) @(negedge clk);

if(out != out_expected) begin

$display("Error - Opcode 0 AND");

$exit;

end

end

// Opcode = 1 functionality

rst = 0;

A = 0;

B = 0;

opcode = 1;

repeat(2) @(negedge clk);

repeat(50) begin

A = $random;

B = $random;

red_op_A = $random;

red_op_B = $random;

if(red_op_A && red_op_B)

out_expected = ^A;

else if(red_op_A)

out_expected = ^A;

else if(red_op_B)

out_expected = ^B;

else

out_expected = A ^ B;

repeat(2) @(negedge clk);

if(out != out_expected) begin

$display("Error - Opcode 1 XOR");

$exit;

end

end

// Opcode = 2 functionality

rst = 0;

A = 0;

B = 0;

opcode = 2;

red_op_A = 0;

red_op_B = 0;

repeat(2) @(negedge clk)

repeat(50) begin

A = $random;

B = $random;

cin = $random;

out_expected = A + B + cin;

repeat(2) @(negedge clk);

if(out != out_expected) begin

$display("Error - Opcode 2 ADD");

$exit;

end

end

// Opcode = 3 functionality

rst = 0;

A = 0;

B = 0;

opcode = 3;

red_op_A = 0;

red_op_B = 0;

repeat(2) @(negedge clk)

repeat(50) begin

A = $random;

B = $random;

out_expected = A * B;

repeat(2) @(negedge clk);

if(out != out_expected) begin

$display("Error - Opcode 3 Multiply");

$exit;

end

end

$display("Simulation Successful");

$display("Testing Invalid Case");

rst = 0;

A = 0;

B = 0;

opcode = 3;

red_op_A = 1;

red_op_B = 0;

repeat(100) @(negedge clk);

$exit;

end // End of Initial Block

endmodule

Simulation:

Figure 1 Testing Reset

Figure 2 Opcode 0

Figure 3 Opcode 1

Figure 4 Opcode 2

Figure 5 Opcode 3

Figure 6 Invalid Case

Linting:

Figure 7 Linting Messages

Do File:

vlib work

vlog ALSU.v ALSU_tb.v

vsim -voptargs=+acc work.alsu_tb

add wave *

add wave alsu_tb/DUT.A_reg

add wave alsu_tb/DUT.red_op_A_reg

add wave alsu_tb/DUT.Invalid_Case

run -all

quit -sim

Vivado:

Constraint File:

## This file is a general .xdc for the Basys3 rev B board

## To use it in a project:

## - uncomment the lines corresponding to used pins

## - rename the used ports (in each line, after get_ports) according to the top level signal names in the project

## Clock signal

set_property -dict {PACKAGE_PIN W5 IOSTANDARD LVCMOS33} [get_ports CLK]

create_clock -period 10.000 -name sys_clk_pin -waveform {0.000 5.000} -add [get_ports CLK]

## Switches

set_property -dict {PACKAGE_PIN V17 IOSTANDARD LVCMOS33} [get_ports {serial_in}]

set_property -dict {PACKAGE_PIN V16 IOSTANDARD LVCMOS33} [get_ports {direction}]

set_property -dict {PACKAGE_PIN W16 IOSTANDARD LVCMOS33} [get_ports {bypass_B}]

set_property -dict {PACKAGE_PIN W17 IOSTANDARD LVCMOS33} [get_ports {bypass_A}]

set_property -dict {PACKAGE_PIN W15 IOSTANDARD LVCMOS33} [get_ports {red_op_B}]

set_property -dict {PACKAGE_PIN V15 IOSTANDARD LVCMOS33} [get_ports {red_op_A}]

set_property -dict {PACKAGE_PIN W14 IOSTANDARD LVCMOS33} [get_ports {cin}]

set_property -dict {PACKAGE_PIN W13 IOSTANDARD LVCMOS33} [get_ports {B[2]}]

set_property -dict {PACKAGE_PIN V2 IOSTANDARD LVCMOS33} [get_ports {B[1]}]

set_property -dict {PACKAGE_PIN T3 IOSTANDARD LVCMOS33} [get_ports {B[0]}]

set_property -dict { PACKAGE_PIN T2    IOSTANDARD LVCMOS33 } [get_ports {A[2]}]

set_property -dict { PACKAGE_PIN R3    IOSTANDARD LVCMOS33 } [get_ports {A[1]}]

set_property -dict { PACKAGE_PIN W2    IOSTANDARD LVCMOS33 } [get_ports {A[0]}]

set_property -dict { PACKAGE_PIN U1    IOSTANDARD LVCMOS33 } [get_ports {opcode[2]}]

set_property -dict { PACKAGE_PIN T1    IOSTANDARD LVCMOS33 } [get_ports {opcode[1]}]

set_property -dict { PACKAGE_PIN R2    IOSTANDARD LVCMOS33 } [get_ports {opcode[0]}]

## LEDs

set_property -dict {PACKAGE_PIN U16 IOSTANDARD LVCMOS33} [get_ports {leds[0]}]

set_property -dict {PACKAGE_PIN E19 IOSTANDARD LVCMOS33} [get_ports {leds[1]}]

set_property -dict {PACKAGE_PIN U19 IOSTANDARD LVCMOS33} [get_ports {leds[2]}]

set_property -dict {PACKAGE_PIN V19 IOSTANDARD LVCMOS33} [get_ports {leds[3]}]

set_property -dict {PACKAGE_PIN W18 IOSTANDARD LVCMOS33} [get_ports {leds[4]}]

set_property -dict {PACKAGE_PIN U15 IOSTANDARD LVCMOS33} [get_ports {leds[5]}]

set_property -dict {PACKAGE_PIN U14 IOSTANDARD LVCMOS33} [get_ports {leds[6]}]

set_property -dict {PACKAGE_PIN V14 IOSTANDARD LVCMOS33} [get_ports {leds[7]}]

set_property -dict { PACKAGE_PIN V13   IOSTANDARD LVCMOS33 } [get_ports {leds[8]}]

set_property -dict { PACKAGE_PIN V3    IOSTANDARD LVCMOS33 } [get_ports {leds[9]}]

set_property -dict { PACKAGE_PIN W3    IOSTANDARD LVCMOS33 } [get_ports {leds[10]}]

set_property -dict { PACKAGE_PIN U3    IOSTANDARD LVCMOS33 } [get_ports {leds[11]}]

set_property -dict { PACKAGE_PIN P3    IOSTANDARD LVCMOS33 } [get_ports {leds[12]}]

set_property -dict { PACKAGE_PIN N3    IOSTANDARD LVCMOS33 } [get_ports {leds[13]}]

set_property -dict { PACKAGE_PIN P1    IOSTANDARD LVCMOS33 } [get_ports {leds[14]}]

set_property -dict { PACKAGE_PIN L1    IOSTANDARD LVCMOS33 } [get_ports {leds[15]}]

##7 Segment Display

#set_property -dict { PACKAGE_PIN W7   IOSTANDARD LVCMOS33 } [get_ports {seg[0]}]

#set_property -dict { PACKAGE_PIN W6   IOSTANDARD LVCMOS33 } [get_ports {seg[1]}]

#set_property -dict { PACKAGE_PIN U8   IOSTANDARD LVCMOS33 } [get_ports {seg[2]}]

#set_property -dict { PACKAGE_PIN V8   IOSTANDARD LVCMOS33 } [get_ports {seg[3]}]

#set_property -dict { PACKAGE_PIN U5   IOSTANDARD LVCMOS33 } [get_ports {seg[4]}]

#set_property -dict { PACKAGE_PIN V5   IOSTANDARD LVCMOS33 } [get_ports {seg[5]}]

#set_property -dict { PACKAGE_PIN U7   IOSTANDARD LVCMOS33 } [get_ports {seg[6]}]

#set_property -dict { PACKAGE_PIN V7   IOSTANDARD LVCMOS33 } [get_ports dp]

#set_property -dict { PACKAGE_PIN U2   IOSTANDARD LVCMOS33 } [get_ports {an[0]}]

#set_property -dict { PACKAGE_PIN U4   IOSTANDARD LVCMOS33 } [get_ports {an[1]}]

#set_property -dict { PACKAGE_PIN V4   IOSTANDARD LVCMOS33 } [get_ports {an[2]}]

#set_property -dict { PACKAGE_PIN W4   IOSTANDARD LVCMOS33 } [get_ports {an[3]}]

##Buttons

set_property -dict {PACKAGE_PIN U18 IOSTANDARD LVCMOS33} [get_ports rst]

#set_property -dict { PACKAGE_PIN T18   IOSTANDARD LVCMOS33 } [get_ports btnU]

#set_property -dict { PACKAGE_PIN W19   IOSTANDARD LVCMOS33 } [get_ports btnL]

#set_property -dict { PACKAGE_PIN T17   IOSTANDARD LVCMOS33 } [get_ports btnR]

#set_property -dict { PACKAGE_PIN U17   IOSTANDARD LVCMOS33 } [get_ports btnD]

##Pmod Header JA

#set_property -dict { PACKAGE_PIN J1   IOSTANDARD LVCMOS33 } [get_ports {JA[0]}];#Sch name = JA1

#set_property -dict { PACKAGE_PIN L2   IOSTANDARD LVCMOS33 } [get_ports {JA[1]}];#Sch name = JA2

#set_property -dict { PACKAGE_PIN J2   IOSTANDARD LVCMOS33 } [get_ports {JA[2]}];#Sch name = JA3

#set_property -dict { PACKAGE_PIN G2   IOSTANDARD LVCMOS33 } [get_ports {JA[3]}];#Sch name = JA4

#set_property -dict { PACKAGE_PIN H1   IOSTANDARD LVCMOS33 } [get_ports {JA[4]}];#Sch name = JA7

#set_property -dict { PACKAGE_PIN K2   IOSTANDARD LVCMOS33 } [get_ports {JA[5]}];#Sch name = JA8

#set_property -dict { PACKAGE_PIN H2   IOSTANDARD LVCMOS33 } [get_ports {JA[6]}];#Sch name = JA9

#set_property -dict { PACKAGE_PIN G3   IOSTANDARD LVCMOS33 } [get_ports {JA[7]}];#Sch name = JA10

##Pmod Header JB

#set_property -dict { PACKAGE_PIN A14   IOSTANDARD LVCMOS33 } [get_ports {JB[0]}];#Sch name = JB1

#set_property -dict { PACKAGE_PIN A16   IOSTANDARD LVCMOS33 } [get_ports {JB[1]}];#Sch name = JB2

#set_property -dict { PACKAGE_PIN B15   IOSTANDARD LVCMOS33 } [get_ports {JB[2]}];#Sch name = JB3

#set_property -dict { PACKAGE_PIN B16   IOSTANDARD LVCMOS33 } [get_ports {JB[3]}];#Sch name = JB4

#set_property -dict { PACKAGE_PIN A15   IOSTANDARD LVCMOS33 } [get_ports {JB[4]}];#Sch name = JB7

#set_property -dict { PACKAGE_PIN A17   IOSTANDARD LVCMOS33 } [get_ports {JB[5]}];#Sch name = JB8

#set_property -dict { PACKAGE_PIN C15   IOSTANDARD LVCMOS33 } [get_ports {JB[6]}];#Sch name = JB9

#set_property -dict { PACKAGE_PIN C16   IOSTANDARD LVCMOS33 } [get_ports {JB[7]}];#Sch name = JB10

##Pmod Header JC

#set_property -dict { PACKAGE_PIN K17   IOSTANDARD LVCMOS33 } [get_ports {JC[0]}];#Sch name = JC1

#set_property -dict { PACKAGE_PIN M18   IOSTANDARD LVCMOS33 } [get_ports {JC[1]}];#Sch name = JC2

#set_property -dict { PACKAGE_PIN N17   IOSTANDARD LVCMOS33 } [get_ports {JC[2]}];#Sch name = JC3

#set_property -dict { PACKAGE_PIN P18   IOSTANDARD LVCMOS33 } [get_ports {JC[3]}];#Sch name = JC4

#set_property -dict { PACKAGE_PIN L17   IOSTANDARD LVCMOS33 } [get_ports {JC[4]}];#Sch name = JC7

#set_property -dict { PACKAGE_PIN M19   IOSTANDARD LVCMOS33 } [get_ports {JC[5]}];#Sch name = JC8

#set_property -dict { PACKAGE_PIN P17   IOSTANDARD LVCMOS33 } [get_ports {JC[6]}];#Sch name = JC9

#set_property -dict { PACKAGE_PIN R18   IOSTANDARD LVCMOS33 } [get_ports {JC[7]}];#Sch name = JC10

##Pmod Header JXADC

#set_property -dict { PACKAGE_PIN J3   IOSTANDARD LVCMOS33 } [get_ports {JXADC[0]}];#Sch name = XA1_P

#set_property -dict { PACKAGE_PIN L3   IOSTANDARD LVCMOS33 } [get_ports {JXADC[1]}];#Sch name = XA2_P

#set_property -dict { PACKAGE_PIN M2   IOSTANDARD LVCMOS33 } [get_ports {JXADC[2]}];#Sch name = XA3_P

#set_property -dict { PACKAGE_PIN N2   IOSTANDARD LVCMOS33 } [get_ports {JXADC[3]}];#Sch name = XA4_P

#set_property -dict { PACKAGE_PIN K3   IOSTANDARD LVCMOS33 } [get_ports {JXADC[4]}];#Sch name = XA1_N

#set_property -dict { PACKAGE_PIN M3   IOSTANDARD LVCMOS33 } [get_ports {JXADC[5]}];#Sch name = XA2_N

#set_property -dict { PACKAGE_PIN M1   IOSTANDARD LVCMOS33 } [get_ports {JXADC[6]}];#Sch name = XA3_N

#set_property -dict { PACKAGE_PIN N1   IOSTANDARD LVCMOS33 } [get_ports {JXADC[7]}];#Sch name = XA4_N

##VGA Connector

#set_property -dict { PACKAGE_PIN G19   IOSTANDARD LVCMOS33 } [get_ports {vgaRed[0]}]

#set_property -dict { PACKAGE_PIN H19   IOSTANDARD LVCMOS33 } [get_ports {vgaRed[1]}]

#set_property -dict { PACKAGE_PIN J19   IOSTANDARD LVCMOS33 } [get_ports {vgaRed[2]}]

#set_property -dict { PACKAGE_PIN N19   IOSTANDARD LVCMOS33 } [get_ports {vgaRed[3]}]

#set_property -dict { PACKAGE_PIN N18   IOSTANDARD LVCMOS33 } [get_ports {vgaBlue[0]}]

#set_property -dict { PACKAGE_PIN L18   IOSTANDARD LVCMOS33 } [get_ports {vgaBlue[1]}]

#set_property -dict { PACKAGE_PIN K18   IOSTANDARD LVCMOS33 } [get_ports {vgaBlue[2]}]

#set_property -dict { PACKAGE_PIN J18   IOSTANDARD LVCMOS33 } [get_ports {vgaBlue[3]}]

#set_property -dict { PACKAGE_PIN J17   IOSTANDARD LVCMOS33 } [get_ports {vgaGreen[0]}]

#set_property -dict { PACKAGE_PIN H17   IOSTANDARD LVCMOS33 } [get_ports {vgaGreen[1]}]

#set_property -dict { PACKAGE_PIN G17   IOSTANDARD LVCMOS33 } [get_ports {vgaGreen[2]}]

#set_property -dict { PACKAGE_PIN D17   IOSTANDARD LVCMOS33 } [get_ports {vgaGreen[3]}]

#set_property -dict { PACKAGE_PIN P19   IOSTANDARD LVCMOS33 } [get_ports Hsync]

#set_property -dict { PACKAGE_PIN R19   IOSTANDARD LVCMOS33 } [get_ports Vsync]

##USB-RS232 Interface

#set_property -dict { PACKAGE_PIN B18   IOSTANDARD LVCMOS33 } [get_ports RsRx]

#set_property -dict { PACKAGE_PIN A18   IOSTANDARD LVCMOS33 } [get_ports RsTx]

##USB HID (PS/2)

#set_property -dict { PACKAGE_PIN C17   IOSTANDARD LVCMOS33   PULLUP true } [get_ports PS2Clk]

#set_property -dict { PACKAGE_PIN B17   IOSTANDARD LVCMOS33   PULLUP true } [get_ports PS2Data]

##Quad SPI Flash

##Note that CCLK_0 cannot be placed in 7 series devices. You can access it using the

##STARTUPE2 primitive.

#set_property -dict { PACKAGE_PIN D18   IOSTANDARD LVCMOS33 } [get_ports {QspiDB[0]}]

#set_property -dict { PACKAGE_PIN D19   IOSTANDARD LVCMOS33 } [get_ports {QspiDB[1]}]

#set_property -dict { PACKAGE_PIN G18   IOSTANDARD LVCMOS33 } [get_ports {QspiDB[2]}]

#set_property -dict { PACKAGE_PIN F18   IOSTANDARD LVCMOS33 } [get_ports {QspiDB[3]}]

#set_property -dict { PACKAGE_PIN K19   IOSTANDARD LVCMOS33 } [get_ports QspiCSn]

## Configuration options, can be used for all designs

set_property CONFIG_VOLTAGE 3.3 [current_design]

set_property CFGBVS VCCO [current_design]

## SPI configuration mode options for QSPI boot, can be used for all designs

set_property BITSTREAM.GENERAL.COMPRESS TRUE [current_design]

set_property BITSTREAM.CONFIG.CONFIGRATE 33 [current_design]

set_property CONFIG_MODE SPIx4 [current_design]

set_property MARK_DEBUG true [get_nets {A_IBUF[2]}]

set_property MARK_DEBUG true [get_nets {leds_OBUF[3]}]

set_property MARK_DEBUG true [get_nets {leds_OBUF[6]}]

set_property MARK_DEBUG true [get_nets {leds_OBUF[12]}]

set_property MARK_DEBUG true [get_nets {opcode_IBUF[0]}]

set_property MARK_DEBUG true [get_nets {opcode_IBUF[2]}]

set_property MARK_DEBUG true [get_nets bypass_A_IBUF]

set_property MARK_DEBUG true [get_nets {leds_OBUF[4]}]

set_property MARK_DEBUG true [get_nets {A_IBUF[0]}]

set_property MARK_DEBUG true [get_nets {A_IBUF[1]}]

set_property MARK_DEBUG true [get_nets red_op_B_IBUF]

set_property MARK_DEBUG true [get_nets red_op_A_IBUF]

set_property MARK_DEBUG true [get_nets bypass_B_IBUF]

set_property MARK_DEBUG true [get_nets direction_IBUF]

set_property MARK_DEBUG true [get_nets {B_IBUF[1]}]

set_property MARK_DEBUG true [get_nets {leds_OBUF[1]}]

set_property MARK_DEBUG true [get_nets {out_OBUF[0]}]

set_property MARK_DEBUG true [get_nets {out_OBUF[3]}]

set_property MARK_DEBUG true [get_nets {leds_OBUF[7]}]

set_property MARK_DEBUG true [get_nets {leds_OBUF[9]}]

set_property MARK_DEBUG true [get_nets {leds_OBUF[11]}]

set_property MARK_DEBUG true [get_nets {leds_OBUF[14]}]

set_property MARK_DEBUG true [get_nets {leds_OBUF[13]}]

set_property MARK_DEBUG true [get_nets {out_OBUF[1]}]

set_property MARK_DEBUG true [get_nets {B_IBUF[0]}]

set_property MARK_DEBUG true [get_nets {B_IBUF[2]}]

set_property MARK_DEBUG true [get_nets cin_IBUF]

set_property MARK_DEBUG true [get_nets {leds_OBUF[0]}]

set_property MARK_DEBUG true [get_nets {leds_OBUF[2]}]

set_property MARK_DEBUG true [get_nets {leds_OBUF[5]}]

set_property MARK_DEBUG true [get_nets {leds_OBUF[15]}]

set_property MARK_DEBUG true [get_nets {out_OBUF[2]}]

set_property MARK_DEBUG true [get_nets {out_OBUF[4]}]

set_property MARK_DEBUG true [get_nets {leds_OBUF[8]}]

set_property MARK_DEBUG true [get_nets {leds_OBUF[10]}]

set_property MARK_DEBUG true [get_nets {opcode_IBUF[1]}]

set_property MARK_DEBUG true [get_nets {out_OBUF[5]}]

set_property MARK_DEBUG true [get_nets clk_IBUF]

set_property MARK_DEBUG true [get_nets rst_IBUF]

set_property MARK_DEBUG true [get_nets serial_in_IBUF]

create_debug_core u_ila_0 ila

set_property ALL_PROBE_SAME_MU true [get_debug_cores u_ila_0]

set_property ALL_PROBE_SAME_MU_CNT 1 [get_debug_cores u_ila_0]

set_property C_ADV_TRIGGER false [get_debug_cores u_ila_0]

set_property C_DATA_DEPTH 1024 [get_debug_cores u_ila_0]

set_property C_EN_STRG_QUAL false [get_debug_cores u_ila_0]

set_property C_INPUT_PIPE_STAGES 0 [get_debug_cores u_ila_0]

set_property C_TRIGIN_EN false [get_debug_cores u_ila_0]

set_property C_TRIGOUT_EN false [get_debug_cores u_ila_0]

set_property port_width 1 [get_debug_ports u_ila_0/clk]

connect_debug_port u_ila_0/clk [get_nets [list clk_IBUF_BUFG]]

set_property PROBE_TYPE DATA_AND_TRIGGER [get_debug_ports u_ila_0/probe0]

set_property port_width 3 [get_debug_ports u_ila_0/probe0]

connect_debug_port u_ila_0/probe0 [get_nets [list {A_IBUF[0]} {A_IBUF[1]} {A_IBUF[2]}]]

create_debug_port u_ila_0 probe

set_property PROBE_TYPE DATA_AND_TRIGGER [get_debug_ports u_ila_0/probe1]

set_property port_width 16 [get_debug_ports u_ila_0/probe1]

connect_debug_port u_ila_0/probe1 [get_nets [list {leds_OBUF[0]} {leds_OBUF[1]} {leds_OBUF[2]} {leds_OBUF[3]} {leds_OBUF[4]} {leds_OBUF[5]} {leds_OBUF[6]} {leds_OBUF[7]} {leds_OBUF[8]} {leds_OBUF[9]} {leds_OBUF[10]} {leds_OBUF[11]} {leds_OBUF[12]} {leds_OBUF[13]} {leds_OBUF[14]} {leds_OBUF[15]}]]

create_debug_port u_ila_0 probe

set_property PROBE_TYPE DATA_AND_TRIGGER [get_debug_ports u_ila_0/probe2]

set_property port_width 3 [get_debug_ports u_ila_0/probe2]

connect_debug_port u_ila_0/probe2 [get_nets [list {opcode_IBUF[0]} {opcode_IBUF[1]} {opcode_IBUF[2]}]]

create_debug_port u_ila_0 probe

set_property PROBE_TYPE DATA_AND_TRIGGER [get_debug_ports u_ila_0/probe3]

set_property port_width 6 [get_debug_ports u_ila_0/probe3]

connect_debug_port u_ila_0/probe3 [get_nets [list {out_OBUF[0]} {out_OBUF[1]} {out_OBUF[2]} {out_OBUF[3]} {out_OBUF[4]} {out_OBUF[5]}]]

create_debug_port u_ila_0 probe

set_property PROBE_TYPE DATA_AND_TRIGGER [get_debug_ports u_ila_0/probe4]

set_property port_width 3 [get_debug_ports u_ila_0/probe4]

connect_debug_port u_ila_0/probe4 [get_nets [list {B_IBUF[0]} {B_IBUF[1]} {B_IBUF[2]}]]

create_debug_port u_ila_0 probe

set_property PROBE_TYPE DATA_AND_TRIGGER [get_debug_ports u_ila_0/probe5]

set_property port_width 1 [get_debug_ports u_ila_0/probe5]

connect_debug_port u_ila_0/probe5 [get_nets [list bypass_A_IBUF]]

create_debug_port u_ila_0 probe

set_property PROBE_TYPE DATA_AND_TRIGGER [get_debug_ports u_ila_0/probe6]

set_property port_width 1 [get_debug_ports u_ila_0/probe6]

connect_debug_port u_ila_0/probe6 [get_nets [list bypass_B_IBUF]]

create_debug_port u_ila_0 probe

set_property PROBE_TYPE DATA_AND_TRIGGER [get_debug_ports u_ila_0/probe7]

set_property port_width 1 [get_debug_ports u_ila_0/probe7]

connect_debug_port u_ila_0/probe7 [get_nets [list cin_IBUF]]

create_debug_port u_ila_0 probe

set_property PROBE_TYPE DATA_AND_TRIGGER [get_debug_ports u_ila_0/probe8]

set_property port_width 1 [get_debug_ports u_ila_0/probe8]

connect_debug_port u_ila_0/probe8 [get_nets [list clk_IBUF]]

create_debug_port u_ila_0 probe

set_property PROBE_TYPE DATA_AND_TRIGGER [get_debug_ports u_ila_0/probe9]

set_property port_width 1 [get_debug_ports u_ila_0/probe9]

connect_debug_port u_ila_0/probe9 [get_nets [list direction_IBUF]]

create_debug_port u_ila_0 probe

set_property PROBE_TYPE DATA_AND_TRIGGER [get_debug_ports u_ila_0/probe10]

set_property port_width 1 [get_debug_ports u_ila_0/probe10]

connect_debug_port u_ila_0/probe10 [get_nets [list red_op_A_IBUF]]

create_debug_port u_ila_0 probe

set_property PROBE_TYPE DATA_AND_TRIGGER [get_debug_ports u_ila_0/probe11]

set_property port_width 1 [get_debug_ports u_ila_0/probe11]

connect_debug_port u_ila_0/probe11 [get_nets [list red_op_B_IBUF]]

create_debug_port u_ila_0 probe

set_property PROBE_TYPE DATA_AND_TRIGGER [get_debug_ports u_ila_0/probe12]

set_property port_width 1 [get_debug_ports u_ila_0/probe12]

connect_debug_port u_ila_0/probe12 [get_nets [list rst_IBUF]]

create_debug_port u_ila_0 probe

set_property PROBE_TYPE DATA_AND_TRIGGER [get_debug_ports u_ila_0/probe13]

set_property port_width 1 [get_debug_ports u_ila_0/probe13]

connect_debug_port u_ila_0/probe13 [get_nets [list serial_in_IBUF]]

set_property C_CLK_INPUT_FREQ_HZ 300000000 [get_debug_cores dbg_hub]

set_property C_ENABLE_CLK_DIVIDER false [get_debug_cores dbg_hub]

set_property C_USER_SCAN_CHAIN 1 [get_debug_cores dbg_hub]

connect_debug_port dbg_hub/clk [get_nets clk_IBUF_BUFG]

Figure 8 Elaboration Schematic

Figure 9 No Critical Warnings

Figure 10 Syntesis Schematic

Figure 11 No Critical Warnings or Errors

Figure 12 Syntehsis Timing Summary

Figure 13 Syntehsis Utilization Summary

Figure 14 Device Snippet

Figure 15 Messages

Figure 16 Timing Summary

Figure 17 Utilization Summary

Q2: Simple DSP:

RTL Code:

module simple_dsp(

A,

B,

C,

D,

clk,

rst_n,

P);

parameter OPERATION = "ADD";

input [17 : 0]  A,

B,

D;

input [47 : 0]  C;

input   clk,

rst_n;

output [47 : 0] P;

reg [17 : 0]    A_reg,

A1_reg,

B_reg,

D_reg,

pre_adder_out;

reg [35 : 0]    multiplier_out;

reg [47 : 0]    C_reg,

P_reg;

// First Stage

always @(posedge clk or negedge rst_n) begin

if(~rst_n) begin

A_reg <= 0;

B_reg <= 0;

C_reg <= 0;

D_reg <= 0;

end

else begin

A_reg <= A;

B_reg <= B;

C_reg <= C;

D_reg <= D;

end

end

// Second Stage

generate

case(OPERATION)

"ADD": begin

always @(posedge clk or negedge rst_n) begin

if(~rst_n) begin

pre_adder_out <= 0;

A1_reg <= 0;

end

else begin

A1_reg <= A_reg;

pre_adder_out <= D_reg + B_reg;

end

end

end

"SUBTRACT": begin

always @(posedge clk or negedge rst_n) begin

if(~rst_n) begin

pre_adder_out <= 0;

A1_reg <= 0;

end

else begin

A1_reg <= A_reg;

pre_adder_out <= D_reg - B_reg;

end

end

end

endcase

endgenerate

// Third Stage

always @(posedge clk or negedge rst_n) begin

if(~rst_n) begin

multiplier_out <= 0;

end

else begin

multiplier_out <= A1_reg * pre_adder_out;

end

end

// Last Stage

generate

case(OPERATION)

"ADD": begin

always @(posedge clk or negedge rst_n) begin

if(~rst_n) begin

P_reg <= 0;

end

else begin

P_reg <= multiplier_out + C_reg;

end

end

end

"SUBTRACT": begin

always @(posedge clk or negedge rst_n) begin

if(~rst_n) begin

P_reg <= 0;

end

else begin

P_reg <= multiplier_out - C_reg;

end

end

end

endcase

endgenerate

assign P = P_reg;

endmodule

Testbench:

module simple_dsp_tb ();

parameter OPERATION = "ADD";

reg [17 : 0]    A,

B,

D;

reg [47 : 0]  C;

reg clk,

rst_n;

wire [47 : 0] P;

simple_dsp  DUT(

A,

B,

C,

D,

clk,

rst_n,

P);

initial begin

clk = 0;

forever begin

#1 clk = ~clk;

end

end

initial begin

rst_n = 0;

repeat(50) begin

A = $random;

B = $random;

C = $random;

D = $random;

repeat(4) @(negedge clk);

if(P != 0) begin

$display("ERROR - Reset");

$stop;

end

end

rst_n = 1;

D = 20;

B = 25;

A = 53;

C = 60;

repeat(4) @(negedge clk);

if(P != 2445) begin

$display("Error - Path 1");

$stop;

end

$display("Simulation Successfull");

$stop;

end

endmodule

Do File:

vlib work

vlog simple_dsp.v simple_dsp_tb.v

vsim -voptargs=+acc work.simple_dsp_tb

add wave *

run -all

quit -sim

Simulation:

Figure 18 Reset Test

Figure 19 Testing Path with fixed Value

Vivado:

Constraint File:

## Clock signal

set_property -dict {PACKAGE_PIN W5 IOSTANDARD LVCMOS33} [get_ports clk]

create_clock -period 10.000 -name sys_clk_pin -waveform {0.000 5.000} -add [get_ports clk]

Figure 20 Elaboration

Figure 21 Nor Critical Warnings or Errors

Figure 22 Synthesis

Figure 23 No Critical Warnings or Errors

Figure 24 Timing Summary

Figure 25 Utilization Summary

Figure 26 Devices Snippet

Figure 27 Timing Summary

Figure 28 Utilization Summary

Q3: Time Division Mux

RTL Code:

module time_mux(in0,

in1,

in2,

in3,

clk,

rst,

out);

input [1 : 0]   in0,

in1,

in2,

in3;

input   rst,

clk;

output reg [1 : 0]  out;

reg [1 : 0] counter_reg;

always @(posedge clk) begin

if(rst)

counter_reg <= 0;

else begin

if(counter_reg >= 3)

counter_reg <= 0;

else

counter_reg <= counter_reg + 1;

end

end

always @(*) begin

case(counter_reg)

0:  out = in0;

1:  out = in1;

2:  out = in2;

3:  out = in3;

endcase

end

endmodule

Testbench:

module time_mux_tb();

reg [1 : 0]     in0,

in1,

in2,

in3;

reg     rst,

clk;

wire [1 : 0]  out;

time_mux DUT(in0,

in1,

in2,

in3,

clk,

rst,

out);

initial begin

clk = 0;

forever begin

#1 clk = ~clk;

end

end

initial begin

in0 = 0;

in1 = 1;

in2 = 2;

in3 = 3;

rst = 1;

@(negedge clk);

if(out != 0) begin

$display("Error - Reset");

$stop;

end

rst = 0;

repeat(1000) @(negedge clk);

$stop;

end

endmodule

Do File:

vlib work

vlog time_mux.v time_mux_tb.v

vsim -voptargs=+acc work.time_mux_tb

add wave *

run -all

quit -sim

Simulation Snippets:

Figure 29 Output Snippet

Vivado:

Constraint File:

## Clock signal

set_property -dict {PACKAGE_PIN W5 IOSTANDARD LVCMOS33} [get_ports clk]

create_clock -period 10.000 -name sys_clk_pin -waveform {0.000 5.000} -add [get_ports clk]

Figure 30 Elaboration Schematic

Figure 31 No Critical Messages or Errors

Figure 32 Synthesis Schematic

Figure 33 No Critical Warnings or Errors

Figure 34 Timing Summary

Figure 35 Utilization Summary

Figure 36 Device Snippet

Figure 37  No Critical Warnings or Errors

Figure 38 Timing Summary

Figure 39 Utilization Summary