Spartan6

DSP48A1

Ameer Ashraf Louly – G3

- RTL Code:

1.1 Mux Reg Module:

module mux_reg(in, out, clk, ce, rst);

parameter WIDTH = 18;

parameter RSTTYPE = "SYNC";

parameter REG = 0;

input [WIDTH - 1 : 0] in;

input   clk,

ce,

rst;

output [WIDTH - 1 : 0] out;

reg [WIDTH - 1 : 0] in_reg;

generate

if(REG == 1) begin

if(RSTTYPE == "SYNC") begin

always @(posedge clk) begin

if(rst)

in_reg <= 0;

else if(ce)

in_reg <= in;

end

end

else if (RSTTYPE == "ASYNC") begin

always @(posedge clk or posedge rst) begin

if(rst)

in_reg <= 0;

else if(ce)

in_reg <= in;

end

end

assign out = in_reg;

end

else if (REG == 0)

assign out = in;

endgenerate

endmodule

  - DSP RTL:

module dsp48a1 #(

parameter   A0REG = 0,

A1REG = 1,

B0REG = 0,

B1REG = 1,

CREG = 1,

DREG = 1,

MREG = 1,

PREG = 1,

CARRYINREG = 1,

CARRYOUTREG = 1,

OPMODEREG = 1,

CARRYINSEL = "OPMODE5",

B_INPUT = "DIRECT",

RSTTYPE = "SYNC"

) (

input [17 : 0]  A,

B,

D,

BCIN,

input [7 : 0]   OPMODE,

input [47 : 0]  C,

PCIN,

input   CLK,

CARRYIN,

RSTA,

RSTB,

RSTM,

RSTP,

RSTC,

RSTD,

RSTCARRYIN,

RSTOPMODE,

CEA,

CEB,

CEM,

CEP,

CEC,

CED,

CECARRYIN,

CEOPMODE,

output  [17 : 0]    BCOUT,

output  [47 : 0]    PCOUT,

P,

output  [35 : 0]    M,

output  CARRYOUT,

CARRYOUTF

);

wire [17 : 0]   A0_out,

D_out,

B0_in,

B0_out,

B1_in,

A1_out,

B1_out;

reg [17 : 0]    pre_adder_subtractor_out;

reg [47 : 0]    x_out,

z_out;

wire [47 : 0]   C_out,

post_adder_subtractor_out,

P_in;

wire [35 : 0]   multiplier_out,

M_out;

wire [7 : 0]    opmode_out;

wire    CYI_in,

CYI_out,

CYO_in;

// Input Stage

mux_reg #(  .REG(A0REG),

.RSTTYPE(RSTTYPE),

.WIDTH(18)) A0_REG (

.in(A),

.out(A0_out),

.clk(CLK),

.ce(CEA),

.rst(RSTA)

);

mux_reg #(  .REG(DREG),

.RSTTYPE(RSTTYPE),

.WIDTH(18)) D_REG (

.in(D),

.out(D_out),

.clk(CLK),

.ce(CED),

.rst(RSTD)

);

// assign B0_in = B;

generate

if(B_INPUT == "DIRECT") begin

assign B0_in = B;

end

else if (B_INPUT == "CASCADE") begin

assign B0_in = BCIN;

end

else begin

assign B0_in = 0;

end

endgenerate

mux_reg #(  .REG(B0REG),

.RSTTYPE(RSTTYPE),

.WIDTH(18)) B0_REG (

.in(B0_in),

.out(B0_out),

.clk(CLK),

.ce(CEB),

.rst(RSTB)

);

mux_reg #(  .REG(OPMODEREG),

.RSTTYPE(RSTTYPE),

.WIDTH(8)) OPMODE_REG (

.in(OPMODE),

.out(opmode_out),

.clk(CLK),

.ce(CEOPMODE),

.rst(RSTOPMODE)

);

mux_reg #(  .REG(CREG),

.RSTTYPE(RSTTYPE),

.WIDTH(48)) C_REG (

.in(C),

.out(C_out),

.clk(CLK),

.ce(CEC),

.rst(RSTC)

);

// Pre Adder Subtractor

always @(*) begin

if(opmode_out[6])

pre_adder_subtractor_out = D_out - B0_out;

else

pre_adder_subtractor_out = D_out + B0_out;

end

assign B1_in = (opmode_out[4]) ? pre_adder_subtractor_out : B0_out;

mux_reg #(  .REG(A1REG),

.RSTTYPE(RSTTYPE),

.WIDTH(18)) A1_REG (

.in(A0_out),

.out(A1_out),

.clk(CLK),

.ce(CEA),

.rst(RSTA)

);

mux_reg #(  .REG(B1REG),

.RSTTYPE(RSTTYPE),

.WIDTH(18)) B1_REG (

.in(B1_in),

.out(B1_out),

.clk(CLK),

.ce(CEB),

.rst(RSTB)

);

assign BCOUT = B1_out; //! Might Need to Change Later

assign multiplier_out = B1_out * A1_out;

mux_reg #(  .REG(MREG),

.RSTTYPE(RSTTYPE),

.WIDTH(36)) M_REG (

.in(multiplier_out),

.out(M_out),

.clk(CLK),

.ce(CEM),

.rst(RSTM)

);

assign M = M_out;

//* Mux X

always @(*) begin

case(opmode_out[1 : 0])

2'b00: x_out = 0;

2'b01: x_out = {12'b000000000000, M_out};

2'b10: x_out = PCOUT;

2'b11: x_out = {D_out[11:0], A1_out[17:0], B1_out[17:0]};

endcase

end

//* Mux Z

always @(*) begin

case(opmode_out[3 : 2])

2'b00: z_out = 0;

2'b01: z_out = PCIN;

2'b10: z_out = PCOUT;

2'b11: z_out = C_out;

endcase

end

generate

if(CARRYINSEL == "CARRYIN")

assign CYI_in = CARRYIN;

else if(CARRYINSEL == "OPMODE5")

assign CYI_in = opmode_out[5];

endgenerate

mux_reg #(  .REG(CARRYINREG),

.RSTTYPE(RSTTYPE),

.WIDTH(1)) CYI_REG (

.in(CYI_in),

.out(CYI_out),

.clk(CLK),

.ce(CECARRYIN),

.rst(RSTCARRYIN)

);

assign {CYO_in, P_in} = (opmode_out[7]) ?

z_out - (x_out + CYI_out) :

x_out + z_out + CYI_out;

mux_reg #(  .REG(PREG),

.RSTTYPE(RSTTYPE),

.WIDTH(48)) P_REG (

.in(P_in),

.out(P),

.clk(CLK),

.ce(CEP),

.rst(RSTP)

);

assign PCOUT = P;

mux_reg #(  .REG(CARRYOUTREG),

.RSTTYPE(RSTTYPE),

.WIDTH(1)) CYO_REG (

.in(CYO_in),

.out(CARRYOUT),

.clk(CLK),

.ce(CECARRYIN),

.rst(RSTCARRYIN)

);

assign CARRYOUTF = CARRYOUT;

endmodule

- Testbench Code:

module dsp48a1_tb();

reg [17 : 0]    A,

B,

D,

BCIN;

reg [7 : 0]     OPMODE;

reg [47 : 0]    C,

PCIN;

reg     CLK,

CARRYIN,

RSTA,

RSTB,

RSTM,

RSTP,

RSTC,

RSTD,

RSTCARRYIN,

RSTOPMODE,

CEA,

CEB,

CEM,

CEP,

CEC,

CED,

CECARRYIN,

CEOPMODE;

wire  [17 : 0]    BCOUT;

wire  [47 : 0]  PCOUT,

P;

wire  [35 : 0]    M;

wire    CARRYOUT,

CARRYOUTF;

dsp48a1 DUT(.A(A),

.B(B),

.D(D),

.BCIN(BCIN),

.OPMODE(OPMODE),

.C(C),

.PCIN(PCIN),

.CLK(CLK),

.CARRYIN(CARRYIN),

.RSTA(RSTA),

.RSTB(RSTB),

.RSTM(RSTM),

.RSTP(RSTP),

.RSTC(RSTC),

.RSTD(RSTD),

.RSTCARRYIN(RSTCARRYIN),

.RSTOPMODE(RSTOPMODE),

.CEA(CEA),

.CEB(CEB),

.CEM(CEM),

.CEP(CEP),

.CEC(CEC),

.CED(CED),

.CECARRYIN(CECARRYIN),

.CEOPMODE(CEOPMODE),

.BCOUT(BCOUT),

.PCOUT(PCOUT),

.P(P),

.M(M),

.CARRYOUT(CARRYOUT),

.CARRYOUTF(CARRYOUTF));

initial begin

CLK = 0;

forever begin

#1 CLK = ~CLK;

end

end

initial begin

// Testing Reset

RSTA = 1;

RSTB = 1;

RSTM = 1;

RSTP = 1;

RSTC = 1;

RSTD = 1;

RSTCARRYIN = 1;

RSTOPMODE = 1;

repeat(100) begin

A = $random;

B = $random;

D = $random;

BCIN = $random;

OPMODE = $random;

C = $random;

PCIN = $random;

CLK = $random;

CARRYIN = $random;

CEA = $random;

CEB = $random;

CEM = $random;

CEP = $random;

CEC = $random;

CED = $random;

CECARRYIN = $random;

CEOPMODE = $random;

@(negedge CLK);

if (BCOUT     != 0 ||

PCOUT     != 0 ||

P         != 0 ||

M         != 0 ||

CARRYOUT  != 0 ||

CARRYOUTF != 0) begin

$display("ERROR - Reset");

$exit;

end

end // End of Reset Test Loop

RSTA = 0;

RSTB = 0;

RSTM = 0;

RSTP = 0;

RSTC = 0;

RSTD = 0;

RSTCARRYIN = 0;

RSTOPMODE = 0;

CEA        = 1;

CEB        = 1;

CEM        = 1;

CEP        = 1;

CEC        = 1;

CED        = 1;

CECARRYIN  = 1;

CEOPMODE   = 1;

OPMODE = 8'b11011101;

A = 20;

B = 10;

C = 350;

D = 25;

repeat(100) begin

BCIN     = $random;

PCIN     = $random;

CARRYIN  = $random;

repeat(4) @(negedge CLK);

if (BCOUT      != 'hf     &&

M          != 'h12c   &&

P          != 'h32    &&

PCOUT      != 'h32    &&

CARRYOUT   != 0       &&

CARRYOUTF  != 0) begin

$display("ERROR - DSP Path 1");

$exit;

end

end // End of DSP Path 1 Loop

OPMODE = 8'b00010000;

A      = 20;

B      = 10;

C      = 350;

D      = 25;

repeat(100) begin

BCIN     = $random;

PCIN     = $random;

CARRYIN  = $random;

repeat(3) @(negedge CLK); // Wait for DREG, B1REG, MREG propagation

if (

BCOUT      != 'h23   &&

M          != 'h2bc  &&

P          != 0      &&

PCOUT      != 0      &&

CARRYOUT   != 0      &&

CARRYOUTF  != 0

)begin

$display("ERROR - DSP Path 2");

$exit;

end

end // End of DSP Path 2 Loop

// DSP Path 3 — No Pre-addition, P Feedback through Mux X and Z

OPMODE = 8'b00001010;

A      = 20;

B      = 10;

C      = 350;

D      = 25;

repeat(100) begin

BCIN     = $random;

PCIN     = $random;

CARRYIN  = $random;

repeat(3) @(negedge CLK); // Wait for B1REG, MREG, PREG propagation

if (

BCOUT      != 'ha   &&

M          != 'hc8  &&

P          != PCOUT &&

CARRYOUT   != CARRYOUTF

) begin

$display("ERROR - DSP Path 3");

$exit;

end

end // End of DSP Path 3 Loop

// DSP Path 4 — Post-subtraction with D:A:B concatenation and PCIN routing

OPMODE = 8'b10100111;

A      = 5;

B      = 6;

C      = 350;

D      = 25;

PCIN   = 3000;

repeat(100) begin

BCIN     = $random;

CARRYIN  = $random;

repeat(3) @(negedge CLK); // Wait for B1REG, MREG, PREG propagation

if (

BCOUT      != 'h6                    &&

M          != 'h1e                   &&

P          != 'hfe6fffec0bb1         &&

PCOUT      != 'hfe6fffec0bb1         &&

CARRYOUT   != 1                      &&

CARRYOUTF  != 1

) begin

$display("ERROR - DSP Path 4");

$exit;

end

end // End of DSP Path 4 Loop

$display("Test Successful");

$exit;

end

endmodule

- Do File:

- Simulation Snippet:

Figure 1 Reset Test

Figure 2 DSP Path 1

Figure 3 DSP Path 2

Figure 4 DSP Path 3

Figure 5 DSP Path 4

- Linting:

Figure 6 Linter Showing no Critical Errors

- Constraint File:

Figure 7 Setting Clock Signal through Constraint File, Nothing else was edited

- Vivado Steps:

7.1 Elaboration:

Figure 8 No Critical Warning or Errors After Elaboration

Figure 9 Elaborated Schematic

  - Synthesis

Figure 10 No Critical Warnings or Errors After Synthesis

Figure 11 Synthesis Timing Report

Figure 12 Synthesis Utilization Report

Figure 13 Synthesized Schematic

  - Implementation:

Figure 14 No Cirtical Warnings or Errors After Implementation

Figure 15 Implementation Timing Report

Figure 16 Implementation Utilization Report

Figure 17 Implemented Device Snippet